PHP: Parsing of messages coming from Basketball Panels and Write into XML

Objective: Write a PHP that receives data from a basketball panel, parses it and writes it into 2 XML files per panel, one only with the last action, the other one with the full event history

Setup (already tested): RS-232/RS-422/RS-485 interface from panel is sending information towards an IOT device that converts the RS-xxx messages into HTTP POST request. The relevant HEX data coming from RS-xxx interface is sent as payload of the HTTP POST request the webserver.

Deliverable: PHP code to be installed on the LAMP server: Ubuntu, Apache 2.4.18, PHP 7.0.22-0
to which you will get access

Droplet Configuration:

Logical Scheme:

RS-232/RS-422 Input Data coming from panel:

- Panel Type Mobatime (RS-232 9600 Baud): see specifications from Bodet (in attachment)

- Panel Type Swiss Timing (RS-422 9600 Baud): see specifications (in attachment)

- Panel Type Stramatel (RS-485 19200 Baud): see specifications (in attachment)

- Panel Type G+D (RS-232, 19200 Baud): see specifications (in attachment)

PHP used as strawman for now (just writing the payload data into a file)

Important: As the panels send data between every 100ms (Mobatime) and 1s (G+D/Stramatel), php code needs to be optimised to only make a write into the XML file if there was a change from the last status for the respective panel, otherwise the data should be discarded. CRC (checksum) - if available - to be checked and packets with invalid CRC to be discarded.

HTTP Request coming from DTU-H100 to Apache Web Server

Current PHP Code on Apache Web Server to test proper reception of request and write log
(for E2E testing purpose, will be replaced by the software you develop)

Output XML Data (last action), for definitions of data structure, see Appendix 1 - 4

|  | SwissTiming (Chapter – Byte) | Mobatime/Bodet (Message – Byte) | Stramatel (Position in 54 byte statement) | G+D (Position in 3x60 char statement) |  |
| --- | --- | --- | --- | --- | --- |
| <document> | - | - | - | - |  |
| <event> | - | - | - | - |  |
| <TeamA>BONCOURT</TeamA> | 4.1 – 3 | n/a, write LOCAL | n/a, write LOCAL | n/a, write LOCAL |  |
| <TeamB>MONTHEY</TeamB> | 4.1 – 15 | n/a, write VISITOR | n/a, write VISITOR | n/a, write VISITOR |  |
| <ScoreTeamA>86</ScoreTeamA> | 3.2 – 8 | 30 – 4 to 6 | 9-11 | Data1/1-3 |  |
| <ScoreTeamB>66</ScoreTeamB> | 3.2 - 11 | 30 – 7 to 9 | 12-14 | Data1/4-6 |  |
| <TeamFoulA>4</TeamFoulA> | 3.2 – 14 | 31 – 5 | 16 | Data1/7-8 |  |
| <TeamFoulB>4</TeamFoulB> | 3.2 – 15 | 31 – 7 | 17 | Data1/12-13 |  |
| <TimeOutA>4</TimeOutA> | 3.2 – 16 | 18 – 9 | 18 | n/a |  |
| <TimeOutB>4</TimeOutB> | 3.2 – 17 | 18 – 10 | 19 | n/a |  |
| <Quarter>Q4</Quarter> | 3.2 – 18 | 18 – 13 | 15 | Data1/10 |  |
| <StartStop>START</StartStop> | 3.2 – 20 | 18 – 3b1 | 21 | Calculated? |  |
| <Timeout>Yes</Timeout> | 3.2 – 22 | 19 – 4 and 5 | 22 | Calculated? |  |
| <ClockTime>00:00</ClockTime> | 3.2 – 3 | 18 – 5 to 8 | 5-8 | Data1/15-19 |  |
| <ClockTimeOut>00</ClockTimeOut> | 3.2 – 22 | 19 – 6 and 7 | 22,47-48 |  |
| <ShotClock>00</ShotClock> | 3.2 - 24 | 50 – 4 and 5 | 53 |  |
| <UTCTime>2017-09-21 21:14:57</UTCTime> | 3.9 – calculated | 20 – calculated (taking date from server) | n/a – calculated (taking date from server) | n/a – calculated (taking date from server) |  |
| </event> | - | - | - | - |  |
| </document> | - | - | - | - |  |

Output XML Data (history), for definitions of data structure, see Appendix 1 - 4

|  | SwissTiming | Mobatime/Bodet | Stramatel | G+D |
| --- | --- | --- | --- | --- |
| <document> | - |  |  |  |
| <playerinfo> | - | - | - | - |
| <PlayerinfoA=NumberOfPlayers> | Calculated | Calculated | Calculated | Calculated |
| <ShirtNo>XX</ShirtNo><Points>XX</Points><Fouls>XX</Fouls> | 3.3 and 3.5 | 37, 58 – 4 to 8, 33 – 4 to 15 | ShirtNo n/a, Points n/a, 23 | Data2/1-2, Data2/4-5, Data2/3 |
| … | - |  |  |  |
| <ShirtNo>XX</ShirtNo><Points>XX</Points><Fouls>XX</Fouls> | 3.3 and 3.5 | 37, 58 – 4 to 8, 33 – 4 to 15 | No shirts, no points, 34 | Data2/56-57, Data2/59-60, Data2/58 |
| </PlayerinfoA> | - |  |  |  |
| <PlayerinfoB=NumberOfPlayers> | Calculated | Calculated | Calculated | Calculated |
| <ShirtNo>XX</ShirtNo><Points>XX</Points><Fouls>XX</Fouls> | 3.4 and 3.6 | 38, 58 – 4 to 8, 33 – 4 to 15 | No shirts, no points, 35 | Data3/1-2, Data3/4-5, Data3/3 |
| … | - |  |  |  |
| <ShirtNo>XX</ShirtNo><Points>XX</Points><Fouls>XX</Fouls> | 3.4 and 3.6 | 38, 58 – 4 to 8, 33 – 4 to 15 | No shirts, no points, 46 | Data3/56-57, Data3/59-60, Data3/58 |
| </PlayerinfoB> | - | - | - | - |
| </playerinfo> | - | - | - | - |

(continued)

| <playbyplay> | - | - | - | - |
| --- | --- | --- | --- | --- |
| <event> | - | - | - | - |
| <TeamA>BONCOURT</TeamA> | 4.1 – 3 | n/a, write LOCAL | n/a, write LOCAL | n/a, write LOCAL |
| <TeamB>MONTHEY</TeamB> | 4.1 – 15 | n/a, write VISITOR | n/a, write VISITOR | n/a, write VISITOR |
| <ScoreTeamA>86</ScoreTeamA> | 3.2 – 8 | 30 – 4 to 6 | 9-11 | 1-3 |
| <ScoreTeamB>66</ScoreTeamB> | 3.2 - 11 | 30 – 7 to 9 | 12-14 | 4-6 |
| <TeamFoulA>4</TeamFoulA> | 3.2 – 14 | 31 – 5 | 16 | 7-8 |
| <TeamFoulB>4</TeamFoulB> | 3.2 – 15 | 31 – 7 | 17 | 12-13 |
| <TimeOutA>4</TimeOutA> | 3.2 – 16 | 18 – 9 | 18 | n/a |
| <TimeOutB>4</TimeOutB> | 3.2 – 17 | 18 – 10 | 19 | n/a |
| <Quarter>Q4</Quarter> | 3.2 – 18 | 18 – 13 | 15 | 10 |
| <StartStop>START</StartStop> | 3.2 – 20 | 18 – 3b1 | 21 | Calculated? |
| <Timeout>Yes</Timeout> | 3.2 – 22 | 19 – 4 and 5 | 22 | Calculated? |
| <ClockTime>00:00</ClockTime> | 3.2 – 3 | 18 – 5 to 8 | 5-8 | 15-19 |
| <ClockTimeOut>00</ClockTimeOut> | 3.2 – 22 | 19 – 6 and 7 | 22,47-48 |  |
| <ShotClock>00</ShotClock> | 3.2 - 24 | 50 – 4 and 5 | 53 |  |
| <UTCTime>2017-09-21 21:14:57</UTCTime> | 3.9 – calculated | 20 – calculated (taking date from server) | 20 – calculated (taking date from server) | 20 – calculated (taking date from server) |
| </event> | - | - | - | - |
| … | - | - | - | - |
| <event> | - | - | - | - |
| As above | - | - | - | - |
| </event> | - | - | - | - |
| </playbyplay> | - | - | - | - |
| </document> | - | - | - | - |

Appendix1: Documentation from SwissTiming

https://www.mouser.ch/ProductDetail/Amphenol-Tuchel/C016-30H006-100-10/?qs=sGAEpiMZZMvG94qpybaIZUbMDiDcULcBqJT0ZjJ9mHY%3d

https://www.mouser.ch/ProductDetail/Amphenol-Tuchel/C016-30D006-100-10/?qs=sGAEpiMZZMvG94qpybaIZUbMDiDcULcBY3rMQk%2f%2fEWE%3d

Appendix2: Documentation Bodet/Mobatime

Appendix3: Documentation Stramatel (incl. Cable Manufacturing)

Wikipedia: A common de-facto standard is the use of: TX+/RX+ or D+ as alternative for B (high for MARK i.e. idle); TX-/RX- or D- as alternative for A (low for MARK i.e. idle)

Appendix4: Documentation G+D